How to Add a New PC to the Network

A quick step-by-step for anyone who needs to connect a new computer to an existing office.

# What you'll need

- A PC or laptop device in Packet Tracer

- An Automatic cable

- Access to the room switch CLI

# Step 1 — Place the PC

- Click End Devices in the bottom toolbar

- Select PC and drag it onto the canvas near the room you want

- Double-click it, go to Config and give it a name like PC_Office_03_New

# Step 2 — Connect it to the switch

- Click the Connections icon in the bottom toolbar

- Choose Automatic cable (the lightning bolt at the top of the list)

- Click the PC and select FastEthernet0

- Click the room switch and pick any free port — for example FastEthernet0/7 if the others are taken

- Wait for the light to go green. Amber just means STP is settling — give it 30 seconds

# Step 3 — Configure the switch port

Open the CLI on the room switch and run these commands. Replace 0/7 with whichever port you used.

enable

configure terminal

interface FastEthernet0/7

switchport mode access

switchport access vlan 10

spanning-tree portfast

no shutdown

exit

end

write memory

Tip: spanning-tree portfast makes the port come up immediately instead of waiting 30 seconds. Only use it on end device ports — never on ports connecting to other switches.

# Step 4 — Set the PC to DHCP

- Double-click the PC

- Go to Desktop tab → IP Configuration

- Click DHCP and wait about 10 seconds

The PC should automatically get an address in the 192.168.10.x range, gateway 192.168.10.1 and DNS 192.168.99.2. If it shows 169.254.x.x instead, the port config wasn't saved — recheck Step 3.

# Step 5 — Test it

Open Command Prompt on the new PC and run these three pings in order:

| Ping | Command | Expected result |
| --- | --- | --- |
| Own gateway | ping 192.168.10.1 | Should pass — PC is on the right VLAN |
| Another VLAN | ping 192.168.40.1 | Should pass — inter-VLAN routing is working |
| Internet | ping 203.0.113.2 | Should pass — NAT and routing are working |

If the first ping fails, check the switch port VLAN assignment. If the third fails but the first two work, check the NAT config on Router_Core.