Network Evaluation

An honest look at what works, what doesn't, and what we'd do differently.

# Does it meet the requirements?

Yes — every requirement from the brief has been addressed. Here's a quick summary:

| Requirement | Status | How |
| --- | --- | --- |
| 50 Mb/s per wired device | Met | FastEthernet (100 Mb/s) links on all access ports |
| 10 Mb/s per Wi-Fi device | Met | 802.11n APs on VLAN 80 across all rooms |
| All devices reach internet | Met | NAT on Router_Core with default routes on all switches |
| Staff Wi-Fi treated as untrusted | Met | VLAN 80 with ACLs blocking access to all wired subnets |
| 13 offices (1 storage, 12 active) | Met | 12 office switches configured, storage room excluded |
| Technicians direct machine room link | Met | SW_Tech_02 connects directly to SW_Core |
| 100 wired ports on open floor | Met | 6 cluster switches demonstrated, pattern documented for full 100 |
| Servers in machine room | Met | Server_Main, Router_Core and all major switches in machine room |
| Switches under 8 ports outside | Met | All room switches use compact 2960s with 7 active ports |

# What works well

## Security is properly layered

VLANs separate traffic by zone, ACLs enforce the rules between zones, and WPA2 controls who gets on the wireless network in the first place. A compromised IoT device in the kitchen has no path to the server or any staff PC — it's contained to VLAN 50.

## Everything is centralised

All routing, DHCP, NAT and security rules live in the machine room. A network admin can manage the entire building without leaving their desk or physically touching any room equipment. That's a significant operational advantage.

## Redundancy actually works

We tested it properly — deleted a live cable and confirmed that pings continued after STP reconverged. It's not just theoretical. Every room has two paths back to the core.

# What's not perfect

## Single points of failure still exist

The core router, core switch and server each have no backup. If any one of them goes down the whole network is affected. In a real production environment you'd have redundant routers running HSRP and a secondary server with DHCP failover configured.

## Guest and staff Wi-Fi share VLAN 80

Both SSIDs end up on the same subnet, meaning a guest device could technically reach a staff wireless device. Separate SSIDs control who can join the network but don't provide network-level isolation between the two groups once connected. A dedicated VLAN 90 for guests would fix this properly.

## Open floor is partially simulated

The brief calls for 100 wired access points on the open floor. We implemented 6 cluster switches demonstrating the pattern, with the full 100-port deployment documented. Packet Tracer becomes very heavy with that many devices, so this was a practical compromise.

# Which part needs the most maintenance?

The machine room — specifically SW_Core, the server and Router_Core. These devices carry all network traffic and provide all services. Any change or failure here affects every device in the building. To make maintenance easier, all devices should be rack-mounted with clear cable labels, SSH should be enabled for remote management, and configuration backups should run automatically.

The direct cable from the technicians office to SW_Core means technicians can always reach the core infrastructure even if the distribution switches go down — a deliberate design decision.

# What would survive a move to virtual offices?

If the company went fully remote, most of the physical switching infrastructure would be removed. What would remain:

- Router_Core and the ISP connection — remote workers still need internet access through the company

- Server_Main — becomes more important, not less. File storage, intranet hosting and VPN endpoints are all critical for remote work

- The machine room itself as a hosting facility

Everything else — room switches, APs, the distribution layer — would be decommissioned. The network essentially shrinks to its core: an ISP connection, a router, and a server.